Cleaning and Maintenance Information

# Accessories

Stair Edgings – Trans-Edge Aluminium

Data Sheet:  (sheet 1 of 2) October 2015  KRO/LB

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product. A regular scheduled maintenance programme should be employed to prevent the build-up of dirt and grease, which will reduce the slip-resistant properties of the profile and detract from the overall appearance of the product.

These guidelines are applicable to Gradus Trans-Edge TEX aluminium stair edgings.

Preventative Maintenance Programme

A regular scheduled maintenance programme should be employed to prevent the build-up of dirt and grease, which will reduce the slip-resistant properties of the channel and detract from the overall appearance of the product.

NB. Under no circumstances should any cleaners containing hydrocarbons, bleach, pine oil or citrus based agents be used to clean any Gradus Stair Edging.

Steam cleaners, floor sealants, polishes or any coatings should not under any circumstances be used on inserts.

Aluminium Channel

Stubborn marks such as boot blacking may be removed from the aluminium channel by means of a green scotch pad and an aluminium safe detergent. Light surface scratches can be removed with careful use of fine wire wool and buffed with a soft cloth.

Insert

Exterior Applications - Twice Weekly or as Required

To remove everyday dirt pressure wash the inserts using a 200 – 250 bar pressure washer with an aluminium safe traffic film remover. Stubborn marks can be removed by scrubbing the inserts with a stiff bristled brush and heavy duty aluminium safe alkaline cleaner.

Interior Applications - Twice Weekly or as Required

Brush down the channel using a soft bristled brush. With clean, warm water, containing  the correct dilution of aluminium safe cleaning solution apply a small amount to a  well rung out damp mop or a Vileda Padmaster/3M Doodlebug and gently clean the surface of the insert in a left to right motion.  During the clean, the water must be changed on a regular basis.  Do not soak the insert. Once the stair edging has been cleaned thoroughly rinse with fresh, clean water, making sure as much water is removed as possible.

Stubborn marks can be removed by scrubbing the inserts with a stiff bristled brush and heavy duty aluminium safe alkaline cleaner.

On completion ensure that the surrounding floorcovering is completely dry before use.

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Vileda Padmaster

3M Doodlebug

Stair Edgings – Trans-Edge Aluminium

Cleaning and Maintenance Information

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

# Accessories

Recommended Cleaning Products for Trans-Edge Aluminium

Gradus has worked with Selden Research to undertake extensive testing on all Gradus stair edgings.  As a result, a range of cleaning products has been recommended for use with Gradus stair edgings in a variety of environments.

The cleaning products listed below are suitable for these particular stair edgings.

For further information about these cleaning products, including environment suitability, instructions for use and COSHH data sheets, please contact:

Selden Research Limited, Staden Business Park, Staden Lane, Buxton, Derbyshire, SK17 9RZ.

Tel: 01298 26226

Email: gradus@selden.co.uk

Web: www.selden.co.uk

| Product Description | Name of Cleaner | Code | Vinyl Floor Safe |
| --- | --- | --- | --- |
| Neutral Cleaners | Neutral Scrubber Drier Detergent | F176 | Yes |
|  | Sabre | C054 | Yes |
| General Purpose Cleaners | Multi Purpose Cleaner | F010 | Yes |
| General Purpose Bactericidal Cleaners | Selgiene | C020 | Yes |
|  | Selgiene Ultra | C066 | Yes |
| Heavy Duty Alkaline Cleaners | H.D. Degreaser | F052 | Yes |
|  | Selclen S | F014 | Yes |
|  | Aluminium Safe TFR* | F071 | No |

*NB. Aluminium Safe TFR (F071) is for external use only

Data Sheet:  (sheet 2 of 2) October 2015  KRO/LB