Installation Guidelines

# Accessories

TEXR1 Trans Edge Heavy Duty

Stair Edging Attachable Ramp

Data Sheet:   (sheet 1 of 1)   September 2013

These installation guidelines apply to the following Gradus profile:

TEXR1

- It should be ensured that all surfaces to which the profile is to be fixed are sound, dry, flat and free from any contaminants.

- Cut the profile to the size required.

- Place the profile in position and mark the position of the screw holes onto the substrate.

- Pilot drill (for timber) or drill the substrate to a depth of 30 - 35mm and insert suitable fixing plugs into the holes.

- Apply cartridge adhesive (in accordance with manufacturer’s instructions) to the underside of the profile.

- Using No. 6 countersunk steel screws, fix through the profile into the plugs and tighten.

Notes:

- The stair edging should not be trafficked until the adhesive has cured.

- When installing stair edgings in an external environment, consideration must be given to drainage for the steps to avoid a pooling of rain water behind the stair edging.

- For exterior applications, an adhesive manufacturer should be consulted to approve a suitable adhesive for external use.

- Stainless steel screws should be used for external applications.

Recommended adhesive products for TEXR1 heavy duty stair edging attachable ramp

Gradus Limited provides this information only as guidance and the legal responsibility for the supply and performance of the adhesives listed below shall remain the sole responsibility of the relevant manufacturer.

By providing this information, Gradus Limited does not imply any equivalents between each of the different manufacturers’ products, or that other products would prove unsatisfactory.  However, use of adhesives not recommended by Gradus should be supported by a guarantee from the adhesive supplier.

Application of the named adhesive must be actioned in strict accordance with the relevant manufacturers’ instructions.

If you have any queries regarding adhesives, please contact the adhesive manufacturer directly.

| Manufacturer | Product Reference | Contact number | Website |
| --- | --- | --- | --- |
| F. Ball and Co. Ltd | F32 / F33 | +44 (0) 1538 361 633 | http://www.f-ball.co.uk/ |
| Tremco | SP060 / MT440 | +44 (0) 1942 251400 | http://www.tremco-illbruck.co.uk/ |
| UZIN | MS Structural | +44 (0) 1788 530 080 | http://www.uzin.co.uk/ |

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

09/13/DT