Gradus is a member of NBS Plus, providing a comprehensive library of pre-written specifications supported by guidance notes. The NBS specification for ELB 31 40 is shown below, plus a detailed Gradus specification and profile description to ensure the correct specification is maintained. For further information on NBS contact NBS Services on 0191 232 9594.

NBS Specification

M50 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

75/ 750/ 550 STAIR NOSINGS AND TRIMS

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: ELB 31 40 with Interior Insert

Material: Polished manganese bronze with PVC inserts.

Colour: Interior Insert: Alpine (LRV: 34.67), Burgundy (LRV: 5.70), Buttercup (LRV: 55.54), Canvas (LRV: 53.27), Cloud (LRV: 28.57), Coffee (LRV: 6.05), Evergreen (LRV: 6.69), Glacier (LRV: 42.09), Ink (LRV: 4.96), Jet (LRV: 4.56), Lavender (LRV: 31.93), Lead (LRV: 12.77), Linen (LRV: 38.59, Midnight (LRV: 6.56), Nutmeg (LRV: 8.15), Ocean (LRV: 8.00), Poppy (LRV: 10.39), Sage (LRV: 29.53), Snowdrift (LRV: 83.77), Steel (LRV: 26.99), Storm (LRV: 14.38), Veranda (LRV: 39.01) (select as required)

Fixing: Mechanical (screw) and adhesive fix – as per manufacturers instructions

Gradus Detailed Specification

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: ELB 31 40

Insert Reference: ELINS40

Insert Width: 40mm

Note: The PVC insert strips must be specified separately.

Gauge: 4mm

Tread Dimension: 50mm

Riser Dimension: 35mm

Channel: Single olished manganese bronze channel with slip-resistant PVC insert

Profile Shape: Kinked Face

Insert Bond: Gradus Fusion advanced adhesive technology

Insert Type & Colour (select as required):

Interior Insert with low risk of slip in interior dry conditions (PTV 68).  Interior Insert in colour: Alpine (LRV: 34.67), Burgundy (LRV: 5.70), Buttercup (LRV: 55.54), Canvas (LRV: 53.27), Cloud (LRV: 28.57), Coffee (LRV: 6.05), Evergreen (LRV: 6.69), Glacier (LRV: 42.09), Ink (LRV: 4.96), Jet (LRV: 4.56), Lavender (LRV: 31.93), Lead (LRV: 12.77), Linen (LRV: 38.59, Midnight (LRV: 6.56), Nutmeg (LRV: 8.15), Ocean (LRV: 8.00), Poppy (LRV: 10.39), Sage (LRV: 29.53), Snowdrift (LRV: 83.77), Steel (LRV: 26.99), Storm (LRV: 14.38), Veranda (LRV: 39.01) (select as required)

Fixing: Mechanical (screw) and adhesive fix – as per manufacturers instructions

Description

Elite Range

A range of attractive and hardwearing stair edgings comprising of bronze channels with fusion bonded slip resistant inserts.  Suitable for interior use, the Gradus Elite bronze range of stair edgings offers a concealed fixing as the inserts are bonded into the channel on site using a specialist double sided tape which is pre-bonded to the insert.  A mechanical lock provided by the use of locating points also provides added reassurance.  Typically used with heavy resilient floorcoverings and carpet from all major manufacturers, Elite bronze stair edgings provide the ideal finish for prestigious locations such as theatres, banks, hotels etc.  As with all Gradus stair edgings ELB 31 40 helps to meet the guidance in Approved Document M (The Buildings Regulations 2000) and helps to achieve an inclusive environment in line with the Disability Discrimination Act (DDA).

# ELB 31 40

Data Sheet:   (sheet 1 of 1)   December 2008

# NBS & Gradus Specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com