Cleaning and Maintenance Information

# Accessories

Stair Edgings – Chromed Aluminium

Data Sheet:   (sheet 1 of 2)   October 2015   KRO/LB

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product. A regular scheduled maintenance programme should be employed to prevent the build-up of dirt and grease, which will reduce the slip-resistant properties of the profile and detract from the overall appearance of the product.

These guidelines are applicable to Gradus chromed aluminium stair edgings. The relevant cleaning method should be selected in line with the stair edging insert.

Daily Cleaning

Brush down the channel using a soft bristled brush and remove dust particles from the insert by wiping with a damp cloth.

Chromed Aluminium Channel

The channel can be cleaned with a good quality chrome cleaner. Manufacturer’s instructions should be followed and care should be taken not to contaminate or damage the surrounding floorcovering.

Inserts

- Interior (Standard and Grained) Inserts

Twice Weekly or as Required

NB. Under no circumstances should any cleaners containing hydrocarbons, bleach, pine oil or citrus based agents be used to clean any Gradus Stair Edging with Interior inserts.

Steam cleaners, power washers, floor sealants, polishes or any coatings should not under any circumstances be used on inserts.

Brush down the channel using a soft bristled brush. With clean, warm water, containing  the correct dilution of a neutral cleaning solution apply a small amount to a  well rung out damp mop or a Vileda Padmaster/3M Doodlebug and gently clean the surface of the insert in a left to right motion.  During the clean, the water must be changed on a regular basis.  Do not soak the insert. Once the stair edging has been cleaned thoroughly rinse with fresh, clean water, making sure as much water is removed as possible.

On completion the staircase should not be used until completely dry.

- Xtra-grip and Xtra-grip Plus Inserts

Twice Weekly or as Required

NB. Under no circumstances should any cleaners containing hydrocarbons, bleach, pine oil or citrus based agents be used to clean any Gradus Stair Edging with Interior inserts.

Steam cleaners, power washers, floor sealants, polishes or any coatings should not under any circumstances be used on inserts.

Brush down the channel using a soft bristled brush. With clean, warm water, containing  the correct dilution of neutral cleaning solution apply a small amount to a  well rung out damp mop or a Vileda Padmaster/3M Doodlebug and gently clean the surface of the insert in a left to right motion.  During the clean, the water must be changed on a regular basis.  Do not soak the insert. Once the stair edging has been cleaned thoroughly rinse with fresh, clean water, making sure as much water is removed as possible

On completion ensure that the surrounding floorcovering is completely dry before use.

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Stair Edgings – Chromed Aluminium

Data Sheet:   (sheet 2 of 2) October 2015   KRO/LB

Cleaning and Maintenance Information

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

# Accessories

Recommended Cleaning Products for Gradus chromed aluminium stair edgings

Gradus has worked with Selden Research to undertake extensive testing on all Gradus stair edgings.  As a result, a range of cleaning products has been recommended for use with Gradus stair edgings in a variety of environments.

The cleaning products listed below are suitable for these particular stair edgings.

For further information about these cleaning products, including environment suitability, instructions for use and COSHH data sheets, please contact:

Selden Research Limited, Staden Business Park, Staden Lane, Buxton, Derbyshire, SK17 9RZ.

Tel: 01298 26226

Email: gradus@selden.co.uk

Web: www.selden.co.uk

| Product Description | Name of Cleaner | Code | Vinyl Floor Safe |
| --- | --- | --- | --- |
| Neutral Cleaners | Neutral Scrubber Drier Detergent | F176 | Yes |
|  | Sabre | C054 | Yes |
| General Purpose Bactericidal Cleaners | Selgiene | C020 | Yes |
|  | Selgiene Ultra | C066 | Yes |