Installation Guidelines

# Accessories

FXT21 / AFT336 - Clip-Top fx

Data Sheet:   (sheet 1 of 1)   29/03/10

Prior to Installation

Ensure that the profile top and base selected are compatible with the floorcovering being used.

Bases with the pre-fix FXB are to be used where the formation of a curve is required.

Bases with the pre-fix AFT are to be used where straight sections are required.

Installation of fx Base

- Ensure that the substrate is clean, dry and free from contaminants.

- Using a trammel or similar, mark the radius required onto the subfloor.

Where FXR 40 is being used always ensure that the fixing tabs are pointing away from the ramped section of the profile.  Where FXR 40 is installed with vinyl always ensure that the base is fixed on top of the vinyl floor.

- Following the line marked on the floor, run a bead of mastic cartridge adhesive along the line for approx. 600mm – 800mm.

- Working from the end, place the base into the adhesive and begin to mechanically fix the base to the floor through the tabs.( No. 6 screw)

- Once the base has been installed and the adhesive has fully dried the top can then be fitted.

- Place one end of the top section onto the “clip” section of the base and then using a rubber mallet strike the top section of the product until a secure fix between top and base is achieved.

- Always overlap the joints of the base sections and top sections

Minimum Radius for Curving FXT21 / FXT31 – 400mm

Minimum Radius for Curving FXR 40 – 400mm (convex) 900mm (concave)

Installation of AFT Base

- Ensure that the substrate is clean, dry and free from contaminants.

- Cut the AFT base to size and apply mastic cartridge adhesive to the back of the profile.

- Fix the base to the substrate in conjunction with mechanical fixings (No.6 screw)

- Cut the top section to size and using a rubber mallet strike the top section until a secure fix is achieved between top and base.

For further information or assistance please contact Gradus Technical Services Department 01625 428922

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com