# TT25M/AFT14

Data Sheet:   (sheet 1 of 1)   18/01/08

# NBS & Gradus Specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Gradus is a member of NBS Plus, providing a comprehensive library of pre-written specifications supported by guidance notes. The NBS specification for TT25M/AFT14 is shown below, plus a detailed Gradus specification and profile description to ensure the correct specification is maintained. For further information on NBS contact NBS Services on 0191 232 9594.

NBS Specification

M50 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

70 / 740 EDGINGS/COVER STRIPS

M51 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

510 EDGINGS/COVER STRIPS

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, Sk11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: TT25M/AFT14

Material: Metallic PVC-u top / Aluminium base

Colour: Burnt Almond, Doubloon, Platinum, Silver, Zinc

Fixing: Base - Mechanical (screw) and adhesive fix – as per manufacturers instructions

Gradus Detailed Specification

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: TT25M/AFT14

Top Shape: Twin

Material: Metallic PVC-u Top / Aluminium base

Gauge: 6mm

Overall Height: 11mm

Width: Top – 27mm / base – 12.5mm

Colour: Burnt Almond, Doubloon, Platinum, Silver, Zinc

Fixing: Base - Mechanical (screw) and adhesive fix – as per manufacturers instructions

Description

Transition Strips – Clip-Top

Clip-Top is a two-piece (three in Bronze) product allowing design flexibility in all commercial situations.  Tops are either PVC-u with co-extruded PVC ribs or bronze.  The tops can be replaced so that the colour can be altered as part of a refurbishment programme or if worn or damaged sections need to be changed, without renewing the base or disrupting the fitted floorcovering.  A number of PVC-u / Metallic PVC-u tops and aluminium bases can be fabricated to a radius shape, subject to approval of template or detailed drawing.