Installation Guidelines

# Accessories

RT52/AFT28 PVC-u Clip-Top

Data Sheet:   (sheet 1 of 1)   February 2013

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

- It should be ensured that all surfaces to which the profile is to be fixed are dry, flat and free from any contaminants.

- Cut the profile top and base to the size required.

- Place the base into position and, through the pre-drilled holes, mark the substrate. Note: When used to ramp down to resilient flooring, Clip-Top bases should always be fitted on top of the resilient floorcovering.

- Drill the marked substrate using a 6mm drill bit and once drilled insert the appropriate sized plug.

- Apply a bead of proprietary cartridge adhesive to the underside of the base section then, ensuring the substrate is free from dust, firmly place the base into position.  On timber subfloors apply adhesive and fix using No6 wood screws.

- Secure in place using a No6 fixing screw.

- Taking the pre-cut top section and holding at a slight angle, align the fixing lugs on the underside of the top section with the base selection.  Using a rubber mallet firmly strike the top section so it secures into the base section.  Continue this process along the profile length until the whole of the top section sits securely in the base.

- Once the installation is complete, check for high spots by running a hand along the top of the profile. If any high spots are detected they can be removed with a firm strike using the rubber mallet.

If any further information is required, please do not hesitate to contact Gradus Technical Support on +44 (0)1625 428922.