# KB 587 H

Data Sheet:   (sheet 1 of 1)   09.08.16

# NBS & Gradus Specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradus.com

Gradus is a member of NBS Plus, providing a comprehensive library of pre-written specifications supported by guidance notes. The NBS specification for KB 587 H is shown below, plus a detailed Gradus specification and profile description to ensure the correct specification is maintained. For further information on NBS contact NBS Services on 0191 232 9594.

NBS Specification

M50 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

70 / 740 EDGINGS/COVER STRIPS

M51 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

510 EDGINGS/COVER STRIPS

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, Sk11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradus.com   Email: sales@gradusworld.com

Reference: KB 587 H

Material: Top – aluminium with wood-effect finish / base – aluminium

Colour: H20 Dark Beech, H22 Light Beech, H30 Light Oak, H40 Maple

Fixing: Base - Mechanical (screw) and adhesive fix – as per manufacturers instructions

Gradus Detailed Specification

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradus.com   Email: sales@gradusworld.com

Reference: KB 587 H

Top Shape: Border

Material: Top – aluminium with wood-effect finish / base – aluminium

Gauge: 10mm – 16mm

Width: Top – 32mm / base – 23mm

Colour: H20 Dark Beech, H22 Light Beech, H30 Light Oak, H40 Maple

Fixing: Base - Mechanical (screw) and adhesive fix – as per manufacturers instructions

Description

Transition Strips – Euro-Clip

Euro-Clip is a range of two-piece wood-effect floor trims for domestic and light traffic commercial use that allows a co-ordinating, safe transition for flooring such as wood, wood laminate and ceramic tiles.  The Euro-Clip system comprises aluminium bases and tops which are clipped into the bases to an adjustable height to suit the floorcoverings.