Installation Guidelines

# Accessories

LTS Luxury Trim System

Data Sheet:   (sheet 1 of 1)   20.08.13DT

Profiles: 		Tops: 	SFBT29,SFBR37

Bases: 	AFT398, AFT3912, AFT3916, AFT708, AFT7012, AFT7016

- It should be ensured that all substrates to which the profile is to be fixed are sound, dry, flat and free from any contaminants.

- Measure and cut the profile top (including PVC-u connector) and aluminium base to the size required.

- Place the base into position and through the pre-drilled holes, mark the substrate.

- a) Concrete/stone/ceramic floors: Drill the marked substrate using a 6mm masonry drill bit and once drilled insert the appropriate sized plug to accommodate a 25mm No6 countersunk wood screw.  Apply a bead of proprietary cartridge adhesive to the underside of the base section then firmly place the base into position. Secure the base in place using a No6 countersunk wood screw.

b) Timber floors: Apply a bead of proprietary cartridge adhesive to the underside of the base section then firmly place the base into position. Secure the base in place using a No6 countersunk wood screw.

- Taking the pre-cut top section, align the fixing leg on the underside of the top section with the base selection.  Using a rubber mallet firmly strike the top section so it secures into the base section.  Continue this process along the profile length until the whole of the top section sits securely in the base. Note: the face of the mallet should be checked for contaminants that could damage the surface finish prior to securing the top in place.

- If any further information is required, please do not hesitate to contact Gradus Technical Services on (+44)1625 428922 or fax on (+44)1625 425226

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com