# ATB33/PBT16

Data Sheet:   (sheet 1 of 1)   12/04/10

# NBS & Gradus Specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Gradus is a member of NBS Plus, providing a comprehensive library of pre-written specifications supported by guidance notes. The NBS specification for ATB33/PBT16 is shown below, plus a detailed Gradus specification and profile description to ensure the correct specification is maintained. For further information on NBS contact NBS Services on 0191 232 9594.

NBS Specification

M50 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

70 / 740 EDGINGS/COVER STRIPS

M51 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

510 EDGINGS/COVER STRIPS

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, Sk11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: ATB33/PBT16

Material: Top - aluminium with anodised metal finish / base – PVC-u

Colour: Polished silver, etched silver

Fixing: Base - Mechanical (screw) and adhesive fix – as per manufacturers instructions

When installed with ceramics, wood or wood laminate flooring a suitable gap must be left between the profile base and the flooring to allow for natural expansion.

Gradus Detailed Specification

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: ATB33/PBT16

Top Shape: Border

Material: Top - aluminium with anodised metal finish / base – PVC-u

Gauge: 12mm – 16mm

Width: Top - 33mm / base – 17.5mm

Colour: Polished silver, etched silver

Fixing: Base - Mechanical (screw) and adhesive fix – as per manufacturers instructions

When installed with ceramics, wood or wood laminate flooring a suitable gap must be left between the profile base and the flooring to allow for natural expansion.

Description

Transition Strips – MTS Multi-Trim System

MTS Multi-Trim System is a versatile and simple solution for common interfaces between floorcoverings that require room for expansion such as wood, wood laminate, ceramics and terrazzo. Available with a choice of PVC-u and Aluminium finishes in a choice of profiles and heights, all of which are self-levelling.  MTS offers a neat and attractive finish to commonly found junctions in commercial applications, and is suitable for use in medium pedestrian and trolley traffic applications such as high street shops, schools, hotels and offices.  A number of profiles can be curved.  Contact Gradus Technical Services for further information.