[image: image1.png]GRADUS



[image: image2.jpg]Gradus Accessories Park Green Macclesfield Cheshire SK117LZ England UK Tel: 01625 428922 Fax: 01625 433949
www.gradusworld.com














It should be ensured that all surfaces to which the profile is to be fixed are dry, flat and free from any contaminants.








Cut the profile top and base to the size required.








Place the base into position and through the pre-drilled holes, mark the substrate.








Drill the marked substrate using a 6mm drill bit and once drilled insert the appropriate sized plug.  (6mm = red plug). 








Apply a bead of proprietary cartridge adhesive to the underside of the base section, then, ensuring the substrate is free from dust, firmly place the base into position.  *On timber subfloors, apply adhesive, and fix using No6 wood screws.  








Secure in place using a No6 fixing screw.








Taking the pre-cut top section, place into position onto the base.  Using the screws supplied, fix the top to the base with a screw driver





If any further information is required, please do not hesitate to contact Gradus Technical Services on (+44)1625 428922 or fax on (+44)1625 425226








Data Sheet:   (sheet 1 of 1)   12/04/10





Accessories





Installation Guidelines





Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949


www.gradusworld.com





KC388AS








