Cleaning and Maintenance Information

# Accessories

EJC 75  Construction Joint Covers

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product:

Preventative Maintenance Programme

A regular scheduled maintenance programme should be employed to prevent the build up of dirt and grease, which will reduce the slip-resistant properties of the channel and detract from the overall appearance of the product.

NB. Under no circumstances should degreasers of any type and floor cleaners/ sanitizers containing hydrocarbons or citrus based agents be used to clean any Gradus Profile.

Floor sealants, polishes or such coating should not under any circumstances be applied to PVC.

The use of an aggressive detergent could result in colour fade within the PVC.

Daily Cleaning

Brush down the profile using a soft bristled brush and remove dust by wiping with a damp cloth.

Twice Weekly or as Required

Brush down the profile using a soft bristled brush. With clean, warm water, containing the correct dilution ratio of ph neutral cleaning solution apply a small amount to a wet green scotch pad and gently clean the surface of the insert in a left to right motion.  During the clean, the water must be changed on a regular basis.  Do not soak the profile. Once the profile has been cleaned thoroughly rinse with fresh, clean water and dry with a dry lint free cloth.

Stubborn marks such as boot blacking may be removed from the PVC profile by means of a green scotch pad and neutral detergent. After cleaning, dry thoroughly with a lint free cloth. Due to the physical properties of the PVC material, we strongly recommend that the green pad is changed regularly.

On completion the profile should be completely dry.

The cleaning methods described for the above mentioned products can be used with other Gradus products that are PVC.

Data Sheet:   (sheet 1 of 1)   04/03/08

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com