Cleaning and Maintenance Information

# Accessories

BCT16 Specialist Floor Trims

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product:

Preventative Maintenance Programme

A regular scheduled maintenance programme prevents build up of dirt and grease, which will reduce the anti-slip properties of the profile and detract from the overall appearance of the product.

Daily Cleaning

Brush down the profiles using a soft bristled brush and remove dust particles from the by wiping with a damp cloth.

Twice Weekly

Brush down the profiles using a soft bristled brush. With clean warm water, apply a small amount of soap-less detergent to a wet green scotch pad and gently clean the surface of the profile.  During the clean, the water must be changed on a regular basis.  Do not soak the profile. Once the profille has been cleaned thoroughly rinse with fresh, clean water and dry with a dry lint free cloth.

Bronze Profiles

To prevent the ageing process associated with all brass/bronze products, the bronze profile should be cleaned regularly with a                proprietary metal polish and then buffed with a lint free cloth.

Data Sheet:   (sheet 1 of 1)   04/03/08

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com