Data Sheet:   (sheet 1 of 1)   25/11/14 LB/DT

# WTS3

# NBS & Gradus Specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Gradus is a member of NBS Plus, providing a comprehensive library of pre-written specifications supported by guidance notes. The NBS specification for TH495 is shown below, plus a detailed Gradus specification and profile description to ensure the correct specification is maintained. For further information on NBS contact NBS Services on 0191 232 9594.

NBS Specification

M50 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

70 EDGINGS/COVER STRIPS / 740 EDGINGS/COVER STRIPS /

M51 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

510 EDGINGS/COVER STRIPS

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, Sk11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: WTS3

Material: Co-extruded pvc

Colour: Clay, Lead, Steel, Canvas, Coffee (select as required)

Fixing: Adhesive fix – as per manufacturers instructions

Gradus Detailed Specification

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: WTS3

Material: Co-extruded pvc

Gauge: 3.25mm

Profile Height: 14mm

Profile Width: Visible top – 40mm / Base - 120mm

Colour: Clay, Lead, Steel, Canvas, Coffee (select as required)

Fixing: Adhesive fix – as per manufacturers instructions

Description

Specialist Floor Trims – Wetroom Transition Strips

Wetroom transition strips are designed to be incorporated into the flooring in wetrooms and shower areas in order to prevent water encroaching into dry areas. They are designed to be slip-resistant in barefoot wet areas and also allow wheelchair access.