- Before installation, ensure that all surfaces to which the profile is to be fitted are dry, flat, free from any debris or contaminants and in an ambient temperature not less than 18°C.

- Cut the profile to the size required.

- Place the profile in position and mark the position of the screw holes onto the substrate.

- Drill the substrate to a depth of 30-35mm and insert suitable fixing plugs into the holes.

- Apply high performance cartridge adhesive with gap filling properties (e.g. Gripfill) to the back of the profile and place onto the substrate. Ensure that all of the profile is in contact with the substrate and there is a minimum 80% coverage of adhesive on the underside of the profile. Please refer to adhesive manufacturers data sheets for application details.

- Using 6mm screws fix through the profile, into the plugs and tighten.

- Clean out the profile channel and install the insert. If the area is colder than 18°C, then the application of heat to the back of the insert may be necessary. Only equipment with controlled heat should be used e.g. hot air welding gun set at a maximum of “5”.

- When using heat care should be taken not to overheat the insert. Only enough heat should be applied that makes the insert more supple NOT hot.

- After the aluminium ramp profile has been secured to the subfloor, cut the insert material to 10mm longer than required (e.g. Width 1500mm / Cut insert at 1510mm / Mark at 755mm).

- Starting from one end of the profile, remove the release paper from the tape on the back of the insert and fit into the profile channel, ensuring the insert is butted tight to the wall.

- Continue fitting the insert into the channel for approx. one third the length of the profile.

- After feeding one third of the insert in from one end, remove the remaining release paper and begin to feed the insert in from the opposing end. Feed the excess back in towards the centre.

- Sweep down the area and remove and dispose of all debris from site.

If any further information is required, please do not hesitate to contact Gradus Technical Services on (+44)1625 428922 or

fax on (+44)1625 433949.