Cleaning and Maintenance Information

# Accessories

FRB026 Matwell Frames

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product:

Preventative Maintenance Programme

A regular scheduled maintenance programme should be employed to prevent the build up of dirt and grease, which will reduce the anti-slip properties of the profile and detract from the overall appearance of the product.

Daily Cleaning

Brush down the profile using a soft bristled brush and remove dust particles by wiping with a damp cloth.

Twice Weekly or as Required

Brush down the profile using a soft bristled brush. With clean, water, apply a small amount of a none soapy detergent to a wet green scotch pad and gently clean the surface of the profile along its width.  During the clean, the water must be changed on a regular basis.  Do not soak the profile. Once the profile has been cleaned thoroughly rinse with fresh, clean water and dry with a dry lint free cloth.

On completion the profile should not be used until completely dry.

Brass Profile

Stubborn marks such as boot blacking may be removed from the brass profile by means of a green scotch pad and neutral detergent. Light surface scratches can be removed with careful use of a proprietary brass cleaner and then buffed with a soft cloth.

Data Sheet:   (sheet 1 of 1)   04/03/11

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com