Installation Guidelines

# Accessories

FRB026 Matwell Frame

Data Sheet:   (sheet 1 of 1)   28/02/08

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

- Ensure that the substrate is sound, dry and free from contaminants.

- Place the profile into position and set the top of the frame so it finishes flush with the top of the opposing floorcovering.  (*See note below)

- Mark the substrate through the predrilled holes.  Drill the substrate using a 6mm drill bit.  Insert plug to suit.

- Apply a proprietary cartridge adhesive to the back of the profile and screw the profile to the floor.

- All corners should be mitre cut.

- Once installed apply smoothing compound to the required level.

* In areas where opposing floorcoverings are not installed, skim the profile to the correct height and level using a spirit level.

If any further information is required, please do not hesitate to contact Gradus Technical Services on (+44)1625 428922 or fax on (+44)1625 425226