Cleaning and Maintenance Information

# Accessories

RAM012 Ramping Frame

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product:

Preventative Maintenance Programme

A regular scheduled maintenance programme should be employed to prevent the build up of dirt and grease, which will reduce the anti-slip properties of the ramp and detract from the overall appearance of the product.

Daily Cleaning

Brush down the ramp using a soft bristled brush and remove dust particles from the insert by wiping with a damp cloth.

Twice Weekly or as Required

Brush down the ramp using a soft bristled brush. With clean, hand warm water, containing the correct dilution ratio of ph neutral cleaning solution apply a small amount to a wet green scotch pad and gently clean the surface of the ramp in a left to right motion.  During the clean, the water must be changed on a regular basis.  Do not soak the ramp. Once the ramp has been cleaned thoroughly rinse with fresh, clean water and dry with a dry lint free cloth.

NB. Under no circumstances should floor cleaners or sanitizers containing citrus based agents or hydrocarbons be used to clean any Gradus PVC product.

Floor sealants, polishes or such coating should not under any circumstances be applied to PVC.

Data Sheet:   (sheet 1 of 1)   04/03/08

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com