# UMFA Matwell Frames

Data Sheet:   (sheet 1 of 1)   18/08/08

8

# NBS & Gradus Specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Gradus is a member of NBS Plus, providing a comprehensive library of pre-written specifications supported by guidance notes. The NBS specification for UMFA Matwell Frames are shown below, plus a detailed Gradus specification and profile description to ensure the correct specification is maintained. For further information on NBS contact NBS Services on 0191 232 9594.

NBS Specification

N10 GENERAL FIXTURES/FURNISHINGS/EQUIPMENT

300 ENTRANCE MATTING / 40 MISCELLANEOUS FITTINGS

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: UMFA12 / UMFA18 / UMFA25 Matwell Frames

Material: Mill Finished Aluminium

Fixing: Adhesive and mechanical (screw) fixing – as per manufacturers guidelines.

Gradus Detailed Specification

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: UMFA12 / UMFA18 / UMFA25 Matwell Frames

Height: UMFA12 - 15mm / UMFA18 – 21mm / UMFA25 – 28mm

Gauge: 3mm

Overall Width: UMFA12 - 21mm / UMFA18 – 15mm / UMFA25 – 21mm

Visible Ramp Width: 3mm

Material: Mill Finished Aluminium

Fixing: Adhesive and mechanical (screw) fixing – as per manufacturers guidelines.

Description

Gradus matwell frames are for use in recessed matwells in conjunction with Gradus primary & secondary barrier matting systems. They provide a smooth, safe transition between a barrier mat and the adjoining floor covering. This protects the edge of the mat from damage and prevents trip accidents caused by uneven flooring. A choice of gauges is available to accommodate mat thicknesses of 12mm (recessed and surface mounted), 18mm and 25mm (recessed only). Matwell frames are offered either in stock lengths or as made to measure kits for individual mat specifications. Gradus’ curving and fabrication capability allows for most mat shapes to be accommodated.