Installation Guidelines

# Safety & Protection Systems

Aluminium Chequer Plates

Data Sheet:   (sheet 1 of 1)   October 2015

ALUMINIUM CHEQUER PLATE

- 2500mm x 1250mm x 2mm (no fixings supplied)

HANDLING

To avoid damage to the surface of aluminium components, some care is needed in handling. This includes:

- Avoid allowing aluminium to scrape against hard or sharp surfaces

- Two people should be used when stacking/unstacking or moving sheets to avoid dragging them over one another

- Do not drag or throw aluminium components

- In order to avoid distortion or damage, use soft slings when lifting heavy components

- Do not walk over sheets whilst moving them

STORAGE

If being stored for extended periods, aluminium should be lightly oiled and stored vertically to ensure air circulation over all surfaces. Aluminium should be stored indoors, in a clean, dry, dust and contaminant free environment and not be in contact with other materials.

GUIDANCE

- Installation and delivery of aluminium components should always be delayed to the last possible moment to avoid accidental staining or damage.

- Newly installed aluminium components most commonly require cleaning due to carelessness with nearby work procedures. This results in staining from such things as mortar, concrete and paints.

- Staining can be avoided or minimized by protecting aluminium surfaces with a clear lacquer or light oil. If the aluminium is tainted with a wet product it should be removed before drying and washed thoroughly with water.

- If dry mortar, plaster or paint needs to be removed from aluminium by scraping, use a plastic or wooden scraper. Metal scrapers will damage the surface of the aluminium.

It is recommended that this sheet should be adhered and screwed to the wall due to its weight and a pattress should also be in place for it to fix to.

Ensure that the substrate is smooth, dry and free from any contaminants such as loose paint, dust and grease.

Drill holes will need to be placed regularly over the sheet and in every corner of each chequer plate prior to installation.

Offer and position the chequer plate to the area to be installed and using a pencil, mark through the holes onto the substrate.

Remove the chequer plate and dependant on the substrate, either pilot drill or bradawl the marked positions.

Apply an “S” bead of proprietary cartridge adhesive to the back of the chequer plate.

Place the chequer plate into position and apply an even pressure over the entire surface of the chequer plate until a satisfactory lie is achieved. Then fix using round head screws.

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com