Installation Guidelines

# Safety & Protection Systems

50mm Sleeved Handrail System

Data Sheet:   (sheet 1 of 2)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Chalk String Line - Tape Measure | - SDS Drill 110V - Impact Wrench 110V - Chop Saw 110V | - Spanner/Ring Spanner 19mm AF - Club Hammer | - 12mm SDS Drill - M12 Socket 19mm AF - 6 and 8mm Allen Impact Driver Bit - 6 and 8mm Allen Keys | - 2 no. M12 x 100mm Thrubolt Galvanised per Post Fixing code: FXG-TB-12-100G |

HANDRAIL COMPONENTS

KS-50-RS-1000Y		2 x 50mm Yellow Sleeved Handrails (1000mm length)

KS-50-RS-1600Y		2 x 50mm Yellow Sleeved Handrails (1600mm length)

KS-50-RS-3200Y		2 x 50mm Yellow Sleeved Handrails (3200mm length)

KS-50-RS-SHORTY	2 x 50mm Yellow Sleeved Handrails (400-1599mm custom length)

KS-50-RS-LONGY		2 x 50mm Yellow Sleeved Handrails (1600-3200mm custom length)

KS-50-EPB		Pre-assembled Black Sleeved Upright End Post (1100mm high)

KS-50-MPB		Pre-assembled Black Sleeved Upright Mid Post (1100mm high)

KS-50-CPB		Pre-assembled Black Sleeved Upright Corner Post (1100mm high)

Fixings for substrate should be ordered separately (2 no. required per post – FXG-TB-12-100G)

SUITABLE SUBSTRATES

- Concrete Only – Slab Grade/Thickness => C25 Concrete / =<100mm Min Thickness

HANDRAILS & SEGREGATION – SINGLE LEVEL APPLICATIONS

- The adopted standard for single ‘ground floor level’ applications of the 50mm Sleeved Handrail System (sleeved) is for;

  - 1600mm upright post centres

  - 550 and 1100mm AFFL Mid and Top rail heights

  - 42mm x 3.2mm C Type Steel Tubular construction

  - Cast steel components

  - All parts hot dip galvanised to BS EN ISO1461

PLACEMENT / SETTING OUT

- For ‘maximum strength’ the upright baseplates should be fixed at 90 degrees to the line of the handrails

- Chalk-line the first fixing hole line for the run of handrails (inset typically 100mm from slab edges and expansion joints)

- Where possible work to 1600mm standard post centres and balance end of runs post centres with appropriate / aesthetically pleasing dimensions typically not less than 1000mm

- Ensure fixing positions are typically 100mm inset from slab edges or expansion joints to avoid cracking edges when drilling and fixing – check with fixings manufacturer for full technical specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Installation Guidelines

# Safety & Protection Systems

50mm Sleeved Handrail System

Data Sheet:   (sheet 2 of 2)   September 2015

CUTTING, DRILLING & FIXING PROCESS

- Handrails can be cut using a 110v chop saw with steel cutting blade (fine tooth blades or thin ‘steel’ grinding discs) - hot work permit may be required

- Using SDS 110v drill rig – drill to approximately 125mm deep using 12mm SDS Drill

- Ensure all debris is cleared from holes, collect-up and dispose

- Place post and knock-in first fixing with club hammer (ensure nuts are run-up to the top of the thread)

- Knock-in second fixing

- Fully tighten with impact wrench using 19 mm AF socket

- Slide through top rails and connect mid rails to fixed post and next post

- Slide over polymer sleeves (cut to length using 110v chop saw and deburr)

- Place, drill and fix next post and repeat to the full handrail run as required

FINISHING

- Ensure all floor fixing anchors are fully tightened – line and level using steel packers to base feet if required to compensate for uneven substrates

- Ensure all grub screws are fully tightened to all components and check overall security and robustness of the handrail installation including a final visual vertical and horizontal level check - adjust as appropriate

- Clean down area and handover on completion

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com