Installation Guidelines

# Safety & Protection Systems

760D / 1100D I Beam Posts (for double rail)

Data Sheet:   (sheet 1 of 2)   September 2015

REQUIREMENTS

| SETTING OUT | - Marker Pen - Tape Measure - Chalk Line / Laser Line |
| --- | --- |
| POWER TOOLS | - SDS Drill 110V /4.5kg - Impact Wrench 110v - 9” Angle Grinder 110v - 4.5” Angle Grinder 110v |
| HAND TOOLS | - Spanner/Ring Spanner 17 and 24mm AF - Club Hammer - 6/8mm Allen Key - Impact Driver |
| DRILLS / BITS / ATTACHMENTS | - M10 Socket 17mm AF - M16 Socket 24mm AF - 9” Angle Grinder Slitting Disc 1mm Thick for Cutting Steel - 4.5” Angle Grinder Flapper Disc 80 Grit for De-Burring - 12/19mm Hole Cutter - 6/8mm Allen Key & or Impact Driver Bit |
| FIXINGS | - Fixings must be ordered separately - FXG-BS16-35G - M16x35 Galvanised Nut, Washer (2) and Bolt Assembly - Rail Lap Joins 4 no. - Safety Ends (Fixings Included 1 no.) - Fishtail Safety Ends 4 no. - Pedestrian Safety Ends 2 no. - FXG-BS16-50G - M16x50 Galvanised Nut, Washer (2) and Bolt Assembly - Rail to Post connections 2 no. - FXG-TB16-150G - M16 x150 Galvanised Thubolt with Nut and Washer - Post to Floor Anchors 4 no. |

BARRIER COMPONENTS FOR 760 I BEAM POST (DOUBLE RAIL)

API-760D		I Beam Post Bolt Down for double rail (760mm high)

API-1100D	I Beam Post Bolt Down for double rail (1100mm high)

AR3-1750	Corrugated Rail (1750mm length)

AR3-3500	Corrugated Rail (3500mm length)

AR3-CC-SHORT	Corrugated Rail – Custom cut to length and holed (0-1749mm)

AR3-CC-LONG	Corrugated Rail – Custom cut to length and holed (1750-350mm)

AR3-3500-R*	Corrugated Rail – Custom curved from 6-50m radius (3500mm length)

Fixings must be ordered separately

SUITABLE BARRIER RAIL SPECIFICATION

- For use with Corrugated Rail – 310 x 85mm

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Installation Guidelines

# Safety & Protection Systems

760D / 1100D I Beam Posts (for double rail)

Data Sheet:   (sheet 2 of 2)   September 2015

PLACEMENT & SETTING OUT

- Bolt-up 2 no. posts to rail using 2 no. M16 x 50 bolt sets and fully tighten

- Stand bolted assembly in position check clearances on equipment, columns, walls being protected and agree formal setting-out dimension for all areas

- Slide-on and bolt-up end terminal to be used and align the end of the rail with fitted end to the required position

- String line the front of baseplates for each run and then mark fixing holes

DRILLING & CUTTING PROCESS

- Using SDS drill rig and 16mm bit – drill 175mm deep, clean out dust from holes

- Make cuts to rails with 9” Angle Grinder – a hot works permit will be required

- Hole rails as required using 19mm hole saw

- Deburr using 5.5” Angle Grinder with Flapper Disc

FIXING PROCESS

- Place fixings through baseplates in drilled holes (ensure nuts are run up to the top of the threads) then knock-in with club hammer and tighten fully with impact wrench (24mm AF)

FINISHING

- Ensure all bolted lap joins and end terminals are fitted correctly and fully tightened

- Clean-down areas and remove all debris and waste from installation areas

BEST PRACTICE TIP

- Ensure all fixings are checked for correctness and fully tightened

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com