Installation Guidelines

# Safety & Protection Systems

Bollard Cover Kits

Data Sheet:   (sheet 1 of 1)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Tape Measure | - SDS Drill 110V/Battery | - Posi Head Screw Driver | - Impact Driver TEK Screw Head Attachment - 8mm - 4mm Masonry drill bit - Posi Head Driver Bit | - Optional tamper-proof security fixing kit required per cover Kit Product code: BA-SFK |

BOLLARD COVER KIT OPTIONS (COLOUR & HEIGHT VARIATIONS)

BCK-168-960-BY		Black & Yellow Bollard Cover Kit (960mm high)

BCK-168-1260-BY		Black & Yellow Bollard Cover Kit (1260mm high)

BCK-168-1560-BY		Black & Yellow Bollard Cover Kit (1560mm high)

BCK-168-960-GW		Green & White Bollard Cover Kit (960mm high)

BCK-168-1260-GW		Green & White Bollard Cover Kit (1260mm high)

BCK-168-1560-GW		Green & White Bollard Cover Kit (1560mm high)

BCK-168-960-RW		Red & White Bollard Cover Kit (960mm high)

BCK-168-1260-RW		Red & White Bollard Cover Kit (1260mm high)

BCK-168-1560-RW		Red & White Bollard Cover Kit (1560mm high)

Fixings for cover kits are optional and should be ordered separately if required

Installation instructions for Steel Core Bollards available separately

ASSEMBLY PROCESS

- Check diameter and height of existing bollards to correctly size the bollard cover kits

- If fitting a non-standard diameter kit a foam core spacer will be supplied with each kit. Check the foam cores are secure within the bollard housing

- Push fit all MID sections together noting the required colour scheme featured in the brochure to be achieved

- Complete with push fit of top cap

- Slide-over Bollard to finish

OPTIONAL TAMPER-PROOF FIXING

- Before fitting top cap - slide the assembled bollard cover kit over bollard (without top cap fitted)

- Then drill and fix the bollard cover kit just above the finished height of the part-fitted kit

  - For steel bollards – drill and fix with provided TEK Screw

  - For steel bollards filled with concrete - the steel core can be drilled through using TEK Screw then additionally drilled using a 4mm masonry bit - insert yellow fixing plug with stainless screw

- Push-fit TOP cap to finish

FINISHING

- An optional 50mm silver reflective band can either be factory fitted or fitted at point of installation

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com