Installation Guidelines

# Safety & Protection Systems

Fishtail Safety Ends

Data Sheet:   (sheet 1 of 1)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Tape Measure | - SDS Drill 110V - Impact Wrench 110v - 9” Angle Grinder 110v - 4.5” Angle Grinder 110v | - Spanner/Ring Spanner 19 and 24mm AF | - 19mm Hole Cutter & Arbour - M16 Socket 24mm AF - 9” Angle Grinder Slitting Disc 1mm Thick for cutting Steel - 4.5” Angle Grinder Flapper disc 80 grit | - M16x35 Galvanised Nut, Washer (2) and Bolt Assembly (x4 fixings per product) Product code: FXG-BS16-35G |

ACCESSORY (SIZE & COLOUR VARIATIONS)

FSE-Y	Fishtail Safety End (Yellow)

FSE-B	Fishtail Safety End (Black)

FSE-LY	Long Fishtail Safety End (Yellow)

FSE-LB	Long Fishtail Safety End (Black)

Fixings must be ordered separately

SUITABLE SPECIFICATIONS

- For use with Corrugated Rail – 310 x 85mm

PLACEMENT / SETTING OUT

- FISHTAIL SAFETY END - Allow +350mm from end of barrier rail when setting out posts and rails to accommodate the Pedestrian Safety

- LONG FISHTAIL SAFETY END - Allow +400mm from end of barrier rail when setting out posts and rails to accommodate the Pedestrian Safety

- Use pre-drilled rail ends for attaching the Fishtail Safety Ends where possible

FIXING PROCESS

- For custom cut barrier end terminations;

  - Cut rail to length – allowing for the length of the Fishtail Safety End using 9” Angle Grinder

  - De-burr using 4.5” Angle Grinder with 80 grit flapper Disc

  - Position Fishtail Safety End over end rail and mark fixing holes (standard rails will have pre-drilled holes, cut rails will need additional holing)

  - Use 110v Drill with 19mm hole saw for holing steel and plastic

- Note – for Long Fishtails the 4 holes will be required to be drilled on-site using 19mm hole saw through plastic in the desired fixing positions

- Place fishtail in position and bolt-up M16 Bolt Sets

- Tighten with Impact Wrench 110v and or Spanner (24mm AF)

FINISHING

- Clean down, remove any marks with hot soapy water

- Repeat to all locations

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com