Installation Guidelines

# Safety & Protection Systems

Lamp Post Protector Kit

Data Sheet:   (sheet 1 of 1)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Tape Measure | - N/A | - Impact Driver - Battery | - 4mm Allen Head Driver Bit or 4mm Allen Key | - Each MID Section has 4 no. M6 Stainless Bolts (Button Security Head included) - Each TOP Section has 2 no. M6 Stainless Bolts (Button Security Head included) |

LAMP POST PROTECTOR KITS (COLOUR OPTIONS)

- LPW-1260-BY	Lamp Post Protector Kit 1260mm High x 280mm Diameter (Black & Yellow)

- LPW-1260-GW	Lamp Post Protector Kit 1260mm High x 280mm Diameter (Green & White)

- LPW-1260-RW	Lamp Post Protector Kit 1260mm High x 280mm Diameter (Red & White)

Fixings included

ASSEMBLY PROCESS

- Check diameter at base and diameter at finished protector height to ensure the lamp-post protector kit is the correct size (designed to fit lamp post up to and including 219mm diameter)

- Check the foam cores are secure within the protector housing and are the correct size

- Start with the base MID Section – remove the 4 Security Bolts using 4mm Allen Key or Impact Driver with 4mm Allen Head Bit

- Open the protector and slide-over the slim part of the lamp-post (above the neck)

- Slide down to base and squeeze together re-fitting 4 no. security bolts

- Repeat to all MID Sections taking note of the colour scheme required (each section will clip to the one below)

- Check TOP cap pre-cut hole is correct size for the lamp post at the finished height – if you require an enlarged hole this can be achieved by scoring and trimming with ‘very sharp’ knife – Take EXTREME CARE when manually trimming and WEAR GLOVES

- Slide the TOP cap over in the same way as the MID’s and fit 2 no. security bolts to finish

FINISHING

- A ‘50mm Reflective Band’ can either be factory fitted or fitted at point of installation

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com