Installation Guidelines

# Safety & Protection Systems

Pedestrian Safety Ends

Data Sheet:   (sheet 1 of 1)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Tape Measure | - SDS Drill 110V - Impact Wrench 110v | - Spanner/Ring Spanner 19 and 24mm AF | - 19mm Hole Cutter & Arbour - M16 Socket 24mm AF - 9” Angle Grinder Slitting Disc 1mm thick for cutting Steel - 4.5” Angle Grinder Flapper disc 80 grit | - M16x35 Galvanised Nut, Washer (2) and Bolt Assembly (x2 fixings per product) Product code: FXG-BS16-35G |

ACCESSORY (SIZE & COLOUR VARIATIONS)

PSE-Y	Pedestrian Safety End (Yellow)

PSE-B	Pedestrian Safety End (Black)

Fixings must be ordered separately

SUITABLE SPECIFICATIONS

- For use with Corrugated Rail – 310 x 85mm

PLACEMENT & SETTING OUT

- Allow +100mm from end of barrier rail when setting out posts and rails to accommodate the Pedestrian Safety

- Use pre-drilled rail ends for attaching the Fishtail Safety Ends where possible

FIXING & CUTTING PROCESS

- Cut rail to length if required using a 9” Angle Grinder 110v

- De-burr with 4.5” angle grinder and slide over Pedestrian Safety End

- If required mark fixing hole positions

- Using Drill with 19mm hole cutter – drill holes in rail

- Ensure all debris is cleared from hole and collect-up and dispose

- Insert M16 Bolt Sets and tighten with 24mm Spanner/Socket/Impact Wrench

FINISHING

- Clean down, remove any marks with hot soapy water

- Repeat to all locations

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com