Installation Guidelines

# Safety & Protection Systems

Protection Post

Data Sheet:   (sheet 1 of 1)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Tape Measure | - SDS Drill 110V - Impact Wrench 110V | - Spanner/Ring Spanner 19mm AF - Club Hammer | - 16mm SDS Drill - M16 Socket - 24mm AF | - M12 x 100mm Thrubolt Galvinised per Post (4 no. included with post) |

PROTECTION POSTS (HEIGHT OPTIONS)

BP-600-Y		Static Post (600mm high)

BP-600-Y		Static Post (800mm high)

BP-1000-Y 	Static Post (1000mm high)

Fixings included, however, site specific fixings may be required – consult with your project team and or call the Gradus technical team to discuss options

SUITABLE SUBSTRATES

- Concrete Only – Slab Grade/Thickness => C25 / =<100mm

PLACEMENT / SETTING OUT

- Typical positioning will be a minimum of 100mm from wall/corner/reveal to edge of baseplate.

- Ensure the post is typically 25mm past the line of traffic / wall to ensure adequate protection from traffic flow

- Place post, mark fixing holes and set aside

DRILLING & FIXING

- Using SDS 110volt Drill rig – drill to approx. 120mm deep

- Clean out holes with blow pump and ensure drilling is vertical in both planes to give a nice flush finish to the fixings

- Ensure all debris is collected and disposed of

- Place post, knock-in M12 fixings as appropriate

- Nuts should be wound to the top so that all the thread is inserted in hole with only nut protruding

- Use club hammer for knocking-in process

- Tighten with Impact Wrench 110v and or Spanner

FINISHING

- Clean down

- Fit black push-fit dust caps

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com