Installation Guidelines

# Safety & Protection Systems

Safety Ends & Corners

Data Sheet:   (sheet 1 of 1)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Tape Measure | - SDS Drill 110V - Impact Wrench 110v | - Spanner/Ring Spanner 17mm AF | - 12mm HSS Drill - M10 Socket 17mm AF - 9” Angle Grinder Slitting Disc 1mm Thick for Cutting Steel Rail - 4.5” Angle Grinder Flapper Disc 80 Grit for De-Burring | - Safety End (1 no.) M10 x 25mm Bolt Set - Safety Corner (2 no.) M10 x 25mm Bolt Set Fixings included |

ACCESSORY (SIZE & COLOUR VARIATIONS)

PSE-Y	Pedestrian Safety End (Yellow)

PSE-B	Pedestrian Safety End (Black)

Fixings included

SUITABLE SPECIFICATIONS

- For use with Corrugated Rail – 310 x 85mm

PLACEMENT / SETTING OUT

- Allow +25mm from end of barrier rail when setting out posts and rails to accommodate the Safety End / Corner

DRILLING & CUTTING PROCESS

- Ends - cut rail flush with what is being protected using a 9” Angle Grinder 110v (this will provide a +25mm protection)

- De-burr using an 80 Grit flapper Disc or similar and slide over Safety End

- Using 110volt or battery drill – drill 12mm diameter hole through the rail at the centre fixing position of the Safety End (pre-drilled)

- Corners – repeat as above and ensure the 2 Safety Ends are set against each other tightly at 90 degrees before drilling and fixing next posts

- Ensure all debris is cleared from hole, collect-up and dispose

FIXING PROCESS

- Insert M10 bolt set – Hex head to the outside and nut/washer to the inside

- Tighten with 17mm socket and 17mm Spanner until the plastic is fully pinched to the barrier rail + half a turn

FINISHING

- Clean down, remove any marks with hot soapy water

- Repeat to all locations

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com