Installation Guidelines

# Safety & Protection Systems

Speed Bumps

Data Sheet:   (sheet 1 of 2)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Chalk String Line - Tape Measure | - SDS Drill 110V - Impact Wrench 110v | - Spanner/Ring Spanner 19mm AF | - 14mm SDS Drill - M12 Socket 19mm AF | - M12 x 100mm Coach Screw + Washer (included with kit) - 14mm x 75mm Nylon Plug - This fixing is deemed standard to suit both concrete and tarmac installations. - Site specific alternatives may be required and should be approved by client/managing agent. - Fixings not included with individual module parts |

SPEED BUMP KITS (LENGTH & HEIGHT OPTIONS)

SBK-3000-50BY	Speed Bump Kit - 3000mm length for Domestic Vehicles (50mm height)

SBK-4000-50BY	Speed Bump Kit - 4000mm length for Domestic Vehicles (50mm height)

SBK-5000-50BY	Speed Bump Kit - 5000mm length for Domestic Vehicles (50mm height)

SBK-6000-50BY	Speed Bump Kit - 6000mm length for Domestic Vehicles (50mm height)

SBK-5000-75BY	Speed Bump Kit - 5000mm length for Commercial Vehicles (75mm height)

SBK-6000-75BY	Speed Bump Kit - 6000mm length for Commercial Vehicles (75mm height)

Fixings included with kit

SPEED BUMP COMPONENT OPTIONS

SB-MID-50Y	Speed Bump MID YELLOW (50mm height)

SB-MID-50B	Speed Bump MID BLACK (50mm height)

SB-END-50Y	Speed Bump END YELLOW (50mm height)

SB-END-50B	Speed Bump END BLACK (50mm height)

SB-MID-75Y	Speed Bump MID YELLOW (75mm height)

SB-MID-75B	Speed Bump MID BLACK (75mm height)

SB-END-75Y	Speed Bump END YELLOW (75mm height)

SB-END-75B	Speed Bump END BLACK (75mm height)

SB-FXG-KZ	Speed Bump Fixing (coach Screw M12 x 100, M12 Washer & Nylon Plug 14mm x 75mm)
		4 no. required per MID section

2 no. required per END section

SUITABLE SUBSTRATES

- Concrete – Slab Grade/Thickness => C25 / =<100mm

- Tarmac – Thickness => 100mm

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Installation Guidelines

# Safety & Protection Systems

Speed Bumps

Data Sheet:   (sheet 2 of 2)   September 2015

PLACEMENT / SETTING OUT

- Ensure the position of speed bump position is on a straight leading-in and leading-out section of the roadway

- Ensure substrate is flat and in good non-cracked condition and avoids any severe undulation

- Set out from the middle of the run and use a chalk line to align

- Place first two sections and abut to the centre point

- Mark fixing holes / ‘spot’ with drill and set aside speed bump sections

- Clean out holes with blow pump and ensure drilling is vertical in both planes to give a nice flush fixing

- Fix down (see below) and repeat process to each section

- Dome End Sections to be fitted at each end where runs do not abut kerbs/walls

- Individual modules are available for bespoke lengths

NOTE – do not overtighten

DRILLING & FIXING

- Using SDS 110volt Drill rig – drill to approx. 125mm deep using 14mm SDS Drill

- Ensure all debris is cleared from holes, collect-up and dispose

- Insert 14mm Nylon Plugs to all pre-drilled holes

- Place speed bump sections and insert coach screws with washers

- Tighten with Impact Wrench 110v

FINISHING

- Repeat to all sections and areas

- Clean down

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com