Installation Guidelines

# Safety & Protection Systems

Steel Core Bollards (bolt down)

Data Sheet:   (sheet 1 of 2)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Tape Measure - Spirit Level | - SDS Drill 110V - Impact Wrench 110V - Vacuum 110V | - Spanner/Ring Spanner 24mm AF/ 32mm AF Blow Pump | - 16mm SDS Drill - M16 Socket 24mm AF - M20 Socket 32mm AF | - Fixings x4 required per Bollard Core (bolt down) Product code: FXG-TB16-150G |

STEEL CORE BOLLARD (HEIGHT OPTIONS)

BCBD-168-900-G		Galvanised Steel Core Bollard (900mm high)

BCBD-168-1200-G		Galvanised Steel Core Bollard (1200mm high)

BCBD-168-1500-G		Galvanised Steel Core Bollard (1500mm high)

Installation instructions for Bollard Cover Kits available separately

SUITABLE SUBSTRATES

- Concrete Only – Slab Grade/Thickness => C25 Concrete / =<150mm Thickness

FIXINGS

- M16 X 150mm Thrubolt

- Options for Chemical Resin Floor Anchors – please request

Site specific fixings may be required - consult with your project team or contact Gradus technical support to discuss options

PLACEMENT / SETTING OUT

- Typical positioning will be a minimum of 300mm from wall/corner/door to edge of the offset baseplate

- Ensure the bollard steel core is typically 25mm past the line of traffic/wall/reveal to ensure adequate protection from traffic flow (with polymer cover fitted this dimension will be 50mm)

- Place bollard, spot drill fixing holes and set aside

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Installation Guidelines

# Safety & Protection Systems

Steel Core Bollards (bolt down)

Data Sheet:   (sheet 2 of 2)   September 2015

DRILLING & FIXING

- Ensure drilling is vertical in both planes to give a nice flush fixing and clean out holes with blow pump

- Using 16mm SDS 110volt drill rig – drill to approx. 175mm deep for M16 x 150 Fixings

- Ensure all debris is cleared from hole and collect-up and dispose

- Blow out holes (blow-pump or vacuum) and final clean down area

- Place bollard & knock-in M16 fixings (ensure nuts are run-up to the top of threads)

- Use club hammer for knocking-in thubolts

- Tighten with Nut Runner/Impact Wrench 110v and or Spanner

FINISHING

- Clean down and fit black push-fit black dust caps

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com