Installation Guidelines

# Safety & Protection Systems

Wall Mounted Posts

Data Sheet:   (sheet 1 of 2)   September  2015

REQUIREMENTS

| SETTING OUT | - Marker Pen - Tape Measure - Chalk Line / Laser Line |
| --- | --- |
| POWER TOOLS | - SDS Drill 110V /4.5kg - Impact Wrench 110v - 9” Angle Grinder 110v - 4.5” Angle Grinder 110v |
| HAND TOOLS | - Spanner/Ring Spanner 17 and 24mm AF - Club Hammer |
| DRILLS / BITS / ATTACHMENTS | - M10 Socket 17mm AF - M16 Socket 24mm AF - 9” Angle Grinder Slitting Disc 1mm Thick for Cutting Steel Armco - 4.5” Angle Grinder Flapper Disc 80 Grit for De-Burring - 12mm HSS Drill - 19mm Hole Cutter - 6/8mm Allen Key & or Impact Driver Bit |
| FIXINGS | - M16 x 180mm Thrubolt 2 no. (included with Wall Mounted Post as part of kit) - FXG-TB16-50G - M16 x 50mm bolt set 1 no. required for Stand-Off Bracket to rail connection - FXG-BS16-35G - M16x35 Galvanised Nut, Washer (2) and Bolt Assembly - Rail Lap Joins 4 no. |

WALL MOUNTED POST COMPONENTS

AWP-50		Wall mounted post – 50mm depth (includes rubber buffer, steel plate and x2 wall anchors)

AWP-70		Wall mounted post – 70mm depth (includes rubber buffer, steel plate and x2 wall anchors)

AWP-SB-200	Stand-Off Bracket (optional) – requires fixings

AR3-1750	Corrugated Rail (1750mm length)

AR3-3500	Corrugated Rail (3500mm length)

AR3-CC-SHORT	Corrugated Rail – Custom cut to length and holed (0-1749mm)

AR3-CC-LONG	Corrugated Rail – Custom cut to length and holed (1750-350mm)

AR3-3500-R*	Corrugated Rail – Custom *curved from 6-50m radius (3500mm length)

Fixings must be ordered separately unless otherwise indicated

SUITABLE BARRIER RAIL SPECIFICATION

- For use with Corrugated Rail – 310 x 85mm

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Installation Guidelines

# Safety & Protection Systems

Wall Mounted Posts

Data Sheet:   (sheet 2 of 2)   September 2015

PLACEMENT & SETTING OUT

- Agree height of rail with project managing agent.  Typical rail centres will be;

  - Domestic Vehicle 450mm AFFL

  - Commercial Vehicle 600mm AFFL

- Select appropriate end terminal and set out from end or corner – mark first fixing hole and chalk-line or laser level the line of the safety barrier run`

DRILLING & CUTTING PROCESS

- Using SDS drill rig and 16mm bit – drill 175mm deep, clean out dust from holes

- Make cuts to rails with 9” Angle Grinder – a hot works permit will be required

- Hole rails as required using 19mm hole saw

- Deburr using 5.5” Angle Grinder with Flapper Disc

FIXING PROCESS

- Place M16 x 180 fixings through wall plates and rubbers (ensure nuts are run up to the top of the threads)

- Additionally bolt-up Stand-Off Bracket if required

- Knock-in with club hammer and tighten fully with impact wrench (24mm AF)

- Repeat to each post location

- Bolt-up rails to wall plates/stand-off bracket

FINISHING

- Ensure all bolted lap joins and end terminals are fitted correctly and fully tightened

- Clean-down areas and remove all debris and waste from installation areas

BEST PRACTICE TIP

- Ensure all fixings are checked for correctness and fully tightened

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com