Installation Guidelines

# Safety & Protection Systems

Wheelstops

Data Sheet:   (sheet 1 of 2)   September 2015

REQUIREMENTS

| SETTING OUT | POWER TOOLS | HAND TOOLS | DRILLS / BITS / ATTACHMENTS | FIXINGS |
| --- | --- | --- | --- | --- |
| - Marker Pen - Chalk String Line - Tape Measure | - SDS Drill 110V - Impact Wrench 110v | - N/A | - 14mm SDS Drill - 15mm SDS Drill - M12 Socket 19mm AF - M16 Socket 24mm AF | - CAR - M12 x 125mm Coach Screw + Washer + 14mm x 75mm Nylon Plug (3 no. included) - LGV – M16 x 125mm Concrete Screw (5 no. included) - HGV – M16 x 125mm Concrete Screw (5 no. included) |

WHEELSTOP (SIZE OPTIONS)

WS-CAR-80	Wheelstop 80mm High x 150mm Wide x 1000mm Long (Domestic Vehicles)

WS-LGV-110	Wheelstop 110mm High x 200mm Wide x 1000mm Long (Double Deckers - Light Goods Vehicles)

WS-HGV-150	Wheelstop 150mm High x 300mm Wide x 1000mm Long (Heavy Goods Vehicles)

Fixings included with Wheelstop

SUITABLE SUBSTRATES

- Concrete – Slab Grade/Thickness => C25 / =<100mm

- Tarmac – Thickness => 100mm

PLACEMENT / SETTING OUT

- Prior to installation review the lowest part of the vehicles / trailers being parked to ensure that the wheelstop height is sized accordingly (ensure suspension and mud flaps clear the top of the wheelstop)

- Agree site specific setting out dimensions - check longest overhang of vehicles using the parking bay – add to this dimension a tolerance/comfort circa 150mm minimum

- Agree if there is to be pedestrian access behind the parking bay – typically this will add approx..1000mm to your setting out dimension out from the middle of the run and use a chalk line to align

WS-CAR-80		        WS-LGV-110			         WS-HGV-150

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Installation Guidelines

# Safety & Protection Systems

Wheelstops

Data Sheet:   (sheet 2 of 2)   September 2015

SPACING

- Using the parking bay white lines agree the spacing between wheelstops – this will typically be width of vehicle 2400mm + 200mm each side so that the wheelstops can be seen when reversing into bay

- Ensure that there is sufficient pedestrian access between bays without creating trip hazard – discuss with client and or managing agent and get approval for the setting out

- Chalk-line the run and place the product to each bay – spot the fixing holes and set-aside for drilling

DRILLING PROCESS

- NOTE – USE THE CORRECT SIZE DRILL – THIS IS CRITICAL FOR SECURE FIXING;

  - CAR – 14mm SDS Drill (Tarmac and Concrete)

  - LGV/HGV – 14mm SDS Drill TARMAC ONLY

  - LGV/HGV – 15mm SDS Drill CONCRETE ONLY

- Using SDS 110 volt Drill rig – drill to approx. 150mm deep

- Ensure all debris is cleared from holes, collect-up and dispose

- Place the product and insert correct fixings for the type of wheel stop

FIXING PROCESS

- CAR – Insert Nylon Plugs, Coach Screws with Washer and tighten using impact wrench with 19mm AF Socket – DO NOT OVERTIGHTEN

- LGV/HGV – Insert Concrete Screws and tighten using impact wrench with 24mm AF socket

FIXING PROCESS

- Clean down area to finish

- NB – Wheelstops can be jet washed periodically to remove surface dirt and debris

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com