Health and Safety Data Sheet

# Accessories

# Section 1: Company identification

Product(s):	Safety Track Primer

Manufacturer:	GRADUS Limited

Park Green

Macclesfield

Cheshire

SK11 7LZ

Section 2: Composition/information on ingredients

Acetone 36-40%, Butone 30-35%

Potentially hazardous ingredients

As above

Section 3: Potential hazards identification

Inhalation:	Harmful

Skin: 		Irritant

Eyes:		Irritant

Ingestion:		Will cause vomiting, nausea and

dizziness

Section 4: First aid measures

Inhalation:	Remove patient to fresh air and seek medical help. If unconscious place in recovery position.

Skin contact:	Wash immediately with soap and water or suitable hand cleaner.

Eye contact:	Flush with water and seek medical attention

Ingestion:	DO NOT induce vomiting. Seek medical attention urgently.

Section 5: Fire fighting measures

Remove uninvolved people from the vicinity of the fire.

Extinguishing media

Dry powder, water mist, foam, carbon dioxide. Do not use water jet.

Protective equipment

In major fire situations, toxic and corrosive vapours will be evolved and self-contained breathing apparatus and acid resistant protective clothing should be worn.

Section 6: Accidental release measures

Extinguish any naked flames or sources of ignition. Allow maximum ventilation to the spillage area (avoid inhaling vapour) and collect spillage into clean container for disposal. Do not allow to enter drains.

7. Handling and storage

Store in cool well ventilated place, away from direct sunlight, at temperatures between 10ºC and 25ºC. Storage is subject to the Petroleum Mixtures Order 1929 and the Highly Flammable Liquids and Gases regulations 1972.

No personal protection is required.

8. Exposure controls/personal protection

Acetone: Long term exposure:	 (8 hours) :

750ppm-1750mg/m²

Short term exposure: (15 minutes)

1600ppm-3550mg/m²

Butone:   Long term exposure:	 (8 hours) :

200ppm-3550mg/m²

Short term exposure: (15 minutes)

300ppm-800mg/m²

9. Physical and chemical properties

Form:		liquid

Colour:		pearlescent

Odour:		pungent

Solubility (water):	Acetone and Butone is soluble. Balance material is insoluble fat.

Decomposition

Temperature:	N/A

10. Stability and reactivity

Highly flammable.

Avoid overheating and contact with oxidising agents.

Auto ignition temperature 465ºC.

11. Toxicological information

LD50 oral (rats): 	Acetone 3.88g/kg

Butone 3.98g/kg

LD50 dermal (rabbits): Acetone >8.0ml/kg

Butone >8.0ml/kg

LC50 inhalation (rats): Not Known

12. Ecological information

Acetone and Butone are readily biodegradeable, however will cause oxygen depletion in water.

13. Disposal considerations

Disposal should be in accordance with local, state or national legislation.

Bury in an authorised landfill site or incinerate under approved controlled conditions.

Dried adhesive can be disposed of in normal waste.

14. Transport information

UN number 1133, packing class group III

15. Regulatory information

R11 highly flammable R36/37 irritating to eyes and respiratory systems.

16. Legislation and other information

Keep containers in well ventilated place. Keep away from sources of ignition. Do not breathe vapour. Avoid contact with eyes. Do not empty into drains. Take precautionary measures against static discharges. Use only in well ventilated areas.

This safety data sheet was prepared in accordance with the CHIP Regulations as implementing (as respects Great Britain) Directive 91/155/EEC.

Safety Track Primer

Data Sheet:   (sheet 1 of 1)   27/02/08

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com