Cleaning and Maintenance Information

# Accessories

Data Sheet:   (sheet 1 of 1)   Feb 2011

MAS97/8 – Anodised Aluminium Metal Skirting

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product:

Preventative Maintenance Programme

A regular scheduled maintenance programme should be employed to prevent the build up of dirt and grease, which will detract from the overall appearance of the product.

Daily Cleaning

Brush down the profiles using a soft bristled brush and remove dust particles by wiping with a damp cloth.

Aluminium Profile

Stubborn marks such as boot blacking may be removed from the aluminium profile by means of a green scotch pad and neutral detergent.

Twice Weekly or as Required

With clean, hand warm water, containing the correct dilution ratio of ph neutral cleaning solution apply a small amount to a wet green scotch pad and gently clean the surface of the skirting in a left to right motion.  During the clean, the water must be changed on a regular basis.  Do not soak the profile. Once the profile has been cleaned thoroughly rinse with fresh, clean water and dry with a dry lint free cloth.

The following should not be used to clean aluminium skirtings:

- Acid or alkaline detergents

- Abrasive products or materials

- Solvents

Detergents should not be applied directly to the surface of the profile and the profile surface should not exceed 30oC or be exposed to direct sunlight.

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com