Installation Guidelines

# Accessories

Carpet Caps

Data Sheet:   (sheet 1 of 1)   March 2015  KRO

These installation guidelines apply to the following Gradus profiles:

CC40, CC75, MCC40, MCC75, ACC40

Carpet caps are used to provide a neat finish where carpet or carpet tile is continued up the wall.

Carpet caps combined with a cove former where required, help form the cove.

The carpet caps should be allowed to acclimatise for 24 hours prior to installation at an ambient temperature of 18o Celsius.

- It should be ensured that all surfaces to which the profile is to be fixed are sound, dry, flat and free from any contaminants.  Wall surfaces should be free from any previous coverings, linings or adhesive residues used to fix them.  Paint films and any mould must be removed and the affected surface treated correctly.

- Mark the position of the top of the carpet cap on the wall and strike a straight line between the two points using a chalk line.

- Cut the profile to the sizes required and mitre cut internal and external corners with either a mitre block and hack saw or radial arm “chop” saw fitted with a suitable blade.

- From the front of the profile, drill 4mmØ pilot holes through the fixing flange, 40mm in from each end and at centres of 300mm.

- Apply one of the recommended adhesives listed below and follow the manufacturer’s instructions for application.

- Lining the top of the profile to the chalk line, place against the wall applying firm pressure along the profile’s length.

- Screw the profile to the wall using a no.6 screw (appropriate plug fixing may be required, suitable for accommodating the substrate) through the pre-drilled holes.

- Ensure that any excess adhesive is removed in line with manufacturers instructions

Recommended adhesive products for carpet caps

Gradus Limited provides this information only as guidance and the legal responsibility for the supply and performance of the adhesives listed below shall remain the sole responsibility of the relevant manufacturer.

By providing this information, Gradus Limited does not imply any equivalents between each of the different manufacturers’ products, or that other products would prove unsatisfactory.  However, use of adhesives not recommended by Gradus should be supported by a guarantee from the adhesive supplier.

Application of the named adhesive must be actioned in strict accordance with the relevant manufacturers’ instructions.

Gradus do not recommend the use of spray adhesives and double sided tapes.

If you have any queries regarding adhesives, please contact the adhesive manufacturer directly.

| Manufacturer | Product Reference | Contact number | Website |
| --- | --- | --- | --- |
| F. Ball and Co. Ltd | F32 / F33 | +44 (0) 1538 361 633 | http://www.f-ball.co.uk/ |
| Tremco | SP060 / MT440 | +44 (0) 1942 251400 | http://www.tremco-illbruck.co.uk/ |
| UZIN | MS Structural | +44 (0) 1788 530 080 | http://www.uzin.co.uk/ |

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

08/13/DT