Installation Guidelines

# Accessories

PVC Skirting’s – Set-In

Data Sheet:   (sheet 1 of 1)   May 2015 KRO/DT

These installation guidelines apply to the following Gradus profiles:

SI 100, SI 100 32, SI 100 50 2, SI 100 50 2.5, SI 100 50 3.2, SI 100 55, SI 150

The set-in skirting should be allowed to acclimatise for 24 hours prior to installation at an ambient temperature of 18o Celsius.

Set-in skirtings are installed prior to the floorcovering.

- It should be ensured that all surfaces to which the profile is to be fixed are sound, dry, flat and free from any contaminants.  Wall surfaces should be free from any previous coverings, linings or adhesive residues used to fix them.  Paint films and any mould must be removed and the affected surface treated correctly.

- Using a piece of skirting as a gauging template, place in position and use a pencil to mark the wall to provide a guideline for adhesive application.

- All internal and external corners should be mitre cut.

- Apply one of the recommended adhesives listed below and follow the manufacturer’s instructions for application.

- Bring both surfaces together and apply even pressure along the skirting ensuring any air bubbles are removed and full contact is made with the wall.

- Vertical joints may be sealed using a mastic seal or PVC cold weld. Joints with the floorcovering may be sealed using a mastic seal, PVC cold weld (or hot weld supplied by the floorcovering manufacturer).

Recommended adhesive products for PVC skirtings – set-in

Gradus Limited provides this information only as guidance and the legal responsibility for the supply and performance of the adhesives listed below shall remain the sole responsibility of the relevant manufacturer.

By providing this information, Gradus Limited does not imply any equivalents between each of the different manufacturers’ products, or that other products would prove unsatisfactory.  However, use of adhesives not recommended by Gradus should be supported by a guarantee from the adhesive supplier.

Application of the named adhesive must be actioned in strict accordance with the relevant manufacturers’ instructions.

Gradus do not recommend the use of spray adhesives and double sided tapes.

If you have any queries regarding adhesives, please contact the adhesive manufacturer directly.

| Manufacturer | Product Reference | Contact number | Website |
| --- | --- | --- | --- |
| F. Ball and Co. Ltd | F60 / F66 | +44 (0) 1538 361 633 | http://www.f-ball.co.uk/ |
| Tremco | SP060 / SF115 | +44 (0) 1942 251400 | http://www.tremco-illbruck.co.uk/ |
| UZIN | WK 222 / KE 2000 S | +44 (0) 1788 530 080 | http://www.uzin.co.uk/ |

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com