Installation Guidelines

# Accessories

STM 75 10  Range Skirting

Data Sheet:   (sheet 1 of 1)   28/02/08

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Please read the following prior to installation of Stratum Skirting

1.  	Prior to installation ensure that the profile is allowed to acclimatise in an ambient temperature of 18 degrees Celsius for 24 hours.

2. 	Ensure that the substrate is dry, smooth and free from contaminants.  Where a substrate is absorbent it may be necessary to prime with a suitable primer.

3.  	Starting in a corner begin to install the profile dry.  Using a radial arm saw, place the skirting face up on the cutting bed of the saw.  Set the saw to the required angle to produce the desired bevel cut on the product.

4.  	Repeat the process by cutting the adjoining piece to produce the reciprocating angle to form a tight fit internal angle.

5.  	Joints between widths should be cut on site. Do not butt joint manufactured ends together as no guarantee on squareness can be given.

6.  	External angles should be bevel cut in the same manner as internal angles.

7.  	Once all sections have been cut, mark the wall at the top of the profile around the whole perimeter of the room.  Apply adhesive to the back of the product and to the wall, taking care not to spread adhesive above the marked line.  Allow the adhesive to become touch dry.

Once dry bring both surfaces together and apply even pressure along the profile to ensure complete contact with the substrate.

8.  	When all the skirting has been adhered to the wall any gaps can be carefully filled and sealed using white mastic sealant or PVC cold weld.

Note.

If any further information is required, please do not hesitate to contact Gradus Technical Services on (+44)1625 428922 or
fax on (+44)1625 425226