Cleaning and Maintenance Information

# Accessories

HXT44 Welded Corner

Data Sheet:   (sheet 1 of 1)   21/01/2015 LB/KRO

This leaflet has been compiled to help the user obtain optimum performance and satisfaction from their selected product:

Preventative Maintenance Programme

A regular scheduled maintenance programme should be employed to prevent the build up of dirt and grease, which could compromise the slip-resistant properties of the adjacent stair edging profile and will detract from the overall appearance of the product.

As Required - PVC-u Profiles

With clean, hand -warm water, containing the correct dilution ratio of cleaning solution if required (see below), gently wipe the profiles using a soft cloth.

Stubborn marks may be removed from the PVC-u side trim/welded corner profiles by means of a green scotch pad and detergent. After cleaning, dry thoroughly with a lint free cloth. Due to the physical properties of the PVC-u material, we strongly recommend that the green pad is changed regularly.

Care should be taken to avoid contamination of adjacent stair edging profiles with water/cleaning detergents. Any contamination should be thoroughly cleaned and the staircase should not be used until completely dry.

Important Note: Under no circumstances should floor cleaners or sanitizers containing citrus based agents or hydrocarbons be used to clean any Gradus Stair Edging or Trim.

Floor sealants, polishes or such coating should not under any circumstances be used to clean side trims/welded corners due to the risk of contamination to the adjacent stair edging profile which will result in the slip-resistance being compromised.

Recommended Cleaning Products for HXT44

Gradus has worked with Selden Research to undertake extensive testing on all Gradus stair trims.  As a result, a range of cleaning products has been recommended for use with Gradus stair profiles.

The cleaning products listed below are suitable for this particular stair trim.

For further information about these cleaning products, including environment suitability, instructions for use and COSHH data sheets, please contact:

Selden Research Limited, Staden Business Park, Staden Lane, Buxton, Derbyshire, SK17 9RZ.

Tel: 01298 26226

Email: gradus@selden.co.uk

Web: www.selden.co.uk

| Product Description | Name | Code | Vinyl Floor Safe |
| --- | --- | --- | --- |
| Neutral Cleaners | Neutral Scrubber Drier Detergent | F176 | Yes |
|  | Sabre | C054 | Yes |
| General Purpose Cleaners | Multi Purpose Cleaner | F010 | Yes |
| General Purpose Bactericidal Cleaners | Selgiene | C020 | Yes |
|  | Selgiene Ultra | C066 | Yes |
| Heavy Duty Alkaline Cleaners | H.D. Degreaser | F052 | Yes |
|  | Selclen S | F014 | Yes |
|  | Triple | F181 | Yes |
| Acid Cleaners | ACT | H005 | Yes |
|  | Selate | H002 | Yes |

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com