Data Sheet:   (sheet 1 of 1)   01/07/10

# Tread Alert

# NBS & Gradus Specifications

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Gradus is a member of NBS Plus, providing a comprehensive library of pre-written specifications supported by guidance notes. The NBS specification for Tread Alert is shown below, plus a detailed Gradus specification and profile description to ensure the correct specification is maintained. For further information on NBS contact NBS Services on 0191 232 9594.

NBS Specification

M50 RUBBER/PLASTICS/CORK/LINO/CARPET TILING/SHEETING

10 / 110 TILING

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: Tread Alert

Material: 	Rubber

Colour: 	Black (LRV: 5.39), Bluebell (LRV: 8.58), Dove (LRV: 28.23), Glacier (LRV 43.10), Canvas (LRV 58.08),

Granite (LRV: 12.60), (select as required)

Fixing: 	Adhesive fix – as per manufacturers instructions

Gradus Detailed Specification

Manufacturer: Gradus Accessories, Park Green, Macclesfield, Cheshire, SK11 7LZ.

Tel: 01625 428922 Fax: 01625 433939

Website: www.gradusworld.com   Email: sales@gradusworld.com

Reference: Tread Alert

Material:	Rubber

Tile Size:	400mm x 400mm

Gauge: 	8.5mm nominal

Options:	Border Tile (TAL B) - 2.5mm thick with a 6mm raised rib at 50mm centres rounded towards one edge of the tile.

Field Tile (TAL F) – butt jointed to form a continuous run.  2.5mm thick with a 6mm raised rib at 50mm centres across the full width of the tile.

Colour: 	Black (LRV: 5.39), Bluebell (LRV: 8.58), Dove (LRV: 28.23), Glacier (LRV 43.10), Canvas (LRV 58.08),

Granite (LRV: 12.60), (select as required)

Fixing: 	Adhesive fix – as per manufacturers instructions

Description

Tread Alert

A rubber tile with raised ribs that acts as tactile surface warning for visually impaired people to recognise a change in level.  Tread Alert has been designed to meet the criteria for tactile surfaces stipulated in the ‘Guidance on the use of Tactile Paving Surfaces’ published by the (then) DETR (1998).  When used correctly Tread Alert helps to ensure the provision of an inclusive environment as recommended in BS8300:2001 and the Joint Committee on Mobility of Blind and Partially Sighted People.

Tread Alert should be used at the approach (top and bottom) of a flight of internal stairs and installed according to the ‘Guidance on the use of Tactile Paving Surfaces’.  The tiles should contrast in colour with the surrounding floorcovering to ensure clear visibility.

Consideration must be given to the frictional characteristics of Tread Alert and the surrounding floorcovering.  Dramatic changes in frictional resistance, such as when used in conjunction with carpet, require special consideration and Tread Alert should be set-in to rubber, vinyl or linoleum to provide a graduated change in surface friction.