Installation Guidelines

# Accessories

SSHR42 Stainless Steel Handrail

Data Sheet:   (sheet 1 of 1)   10/7/13

Gradus Accessories, Park Green, Macclesfield, Cheshire SK11 7LZ England UK  Tel: 01625 428922  Fax: 01625 433949

www.gradusworld.com

Please read the following prior to installation of Gradus SSHR42 Stainless Steel Handrails

- Establish the fitting height above FFL (Finished Floor Level) to top of rail.

- Set laser level 132mm below established height and position bottom edge of bracket on this line.

- Once in position mark the substrate through the fixing holes in the rail bracket.

- Fix brackets to wall using stainless steel screws and fixings suitable for the substrate. Place brackets 100mm from the ends and at intervals not exceeding 700mm

- Accurately measure each section of rail making allowance for returns, internal and external corners.

- Cut to size using a chop saw fitted with a specialist carbide or cermet tipped blade designed to cut stainless steel. Making sure that tube is clamped and square to blade. Care should be taken when cutting the stainless steel tube to avoid heat damage to the finish.

- De-burr and polish if necessary. Position rail on brackets, transfer position of fixing holes onto rail. Remove rail and drill holes. Reposition rail and fix to brackets with self-tapping stainless steel countersunk screws.

- Push fit returns or corners into the tube using liner joints and a proprietary epoxy structural adhesive.